PE2SIM.EXE is interesting for Chemical researchers and engineers that work in the field of polyester or polyamide synthesis.

This DOS application is freeware

The software package contains the following files:

PE2SIM.EXE	: The simulator application

PE2SIM.DOCX	: This file, description and manual

Pes_csv2.xlsm	: Macro, to enable viewing results of PE2SIM in Excel

SOURCE.ZIP	: Source code, free to change

EXAMPL.ZIP	: Examples in .MLD format to load in PE2SIM.EXE

README.TXT     : Read this file before you start

The report below was written in 1988 by me (Hans Hendriks). It describes in detail how PE2SIM.EXE works and what it produces: a “Monte Carlo” based simulator that produces molecular weight distributions originating from polycondensation reactions.

The written content in the reports Introduction section was a correction added by a reviewer. It says that the Gel constant (“K”) originates from Carothers and not from Flory. This correction is right.

The simulator has undergone a major upgrade in 2021.

Next to the features described in the report, some were changed and new features were added:

- A convertor from generated data to “comma separated files” (.CSV format) for easy displaying in other programs like spreadsheets, was added. In this software package, together with PE2SIM.EXE, the Excel macro “Pes_csv2.xlsm” is included. With this macro, extra features can be observed, like a movie playing the development of molweight distributions, and more.

- Navigation keys were changed to make use on a phone simpler: additional to the cursor keys, it is possible to use the letter keys (l=left, r=right, u=up, d=down). The F-key navigation, described in the report below, is replaced by number-key navigation in this upgraded version. Running PE2SIM on a phone or tablet can be done with use of a DOS simulator app like “Dosbox”.

- Results can be saved as .MLD files: formerly mentioned in this old report as “.MOL“ files. MLD files can only be read by the simulator. A .CSV file can only be generated, if the results are saved in .MLD format first.

- To get a better approach with reality, the weight average Mw is not calculated anymore by the Stockmayer equation, as mentioned in the old report. The calculation of Mw is now done by processing of the weights generated by the simulator. The disadvantage is that during the collision process, it seems as if running of the program halts several times: this is because time is consumed to calculate the Weight Average Mw during the simulation performance.